|  | Talbot Tool Company Ltd. Tel: +44 (0)1273 508881 Fax: +44 (0)1273 540544 |
| --- | --- |
| TALBOT MARKING/LAYOUT FLUID SAFETY DATA SHEET Issue No.4 Date: 25th September 2003 |  |

| 1. IDENTIFICATION OF THE SUBSTANCE/PREPARATION AND COMPANY |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PRODUCT NAME: |  |  | Talbot Blue/White/Green/Red/Yellow |  |  |  |  |
| CHEMICAL NAME: |  |  | Denatured Ethanol |  |  |  |  |
| MANUFACTURER/SUPPLIER: |  |  | Talbot Tool Co. Ltd. |  |  |  |  |
|  |  |  | Crowhurst Road |  |  |  |  |
|  |  |  | Brighton |  |  |  |  |
|  |  |  | BN1 8AT |  |  |  |  |
| TELEPHONE NO: |  |  | 01273 508881 |  |  |  |  |
| FAX NO: |  |  | 01273 540544 |  |  |  |  |
| URL: |  |  | www.talbot-tool.co.uk |  |  |  |  |
| 2. COMPOSITION/INFORMATION ON INGREDIENTS |  |  |  |  |  |  |  |
| INGREDIENT NAME |  |  | CAS No. | CONTENTS | EINECS No. | RISK |  |
|  |  |  |  |  |  | (R No.) |  |
| ETHANOL |  |  | 64-17-5 | 60-100% | 200-578-6 | 11 |  |
| METHANOL |  |  | 67-56-1 | 1-5% | 200-659-6 | 11/23/24/25/39/23/24/25 |  |
| SHELLAC |  |  | - | 1-5% | - | - |  |
| DYE POWDER |  |  | - | <1% | - | - |  |
| 3. HAZARDS IDENTIFICATION |  |  |  |  |  |  |  |
| Highly flammable |  |  |  |  |  |  |  |
| Harmful by inhalation, in contact with skin and if swallowed |  |  |  |  |  |  |  |
| Harmful: possible risk of irreversible effects through inhalation, in contact with skin and if swallowed. |  |  |  |  |  |  |  |
| 4. FIRST AID MEASURES |  |  |  |  |  |  |  |
| GENERAL: |  |  | Move the exposed person to fresh air at once. Get medical attention if any |  |  |  |  |
|  |  |  | discomfort continues. |  |  |  |  |
| INHALATION: |  |  | Move the exposed person to fresh air at once. Perform artificial respiration if |  |  |  |  |
|  |  |  | breathing has stopped. When breathing is difficult, properly trained personnel may |  |  |  |  |
|  |  |  | assist affected person by administering 100% oxygen. Keep the affected person |  |  |  |  |
|  |  |  | warm and at rest. Get prompt medical attention. |  |  |  |  |
| INGESTION: |  |  | NEVER MAKE AN UNCONSCIOUS PERSON VOMIT OR DRINK FLUIDS! |  |  |  |  |
|  |  |  | Drink plenty of water. DO NOT induce vomiting. Get medical attention immediately. |  |  |  |  |
| SKIN: |  |  | Remove affected person from source of contamination. Promptly wash |  |  |  |  |
|  |  |  | contaminated skin with soap or mild detergent and water. Promptly remove |  |  |  |  |
|  |  |  | clothing if soaked through and wash as above. Get medical attention if irritation |  |  |  |  |
|  |  |  | persists after washing. |  |  |  |  |
| EYES: |  |  | Promptly wash eyes with plenty of water while lifting the eye lids. Continue to rinse |  |  |  |  |
|  |  |  | for at least 15 minutes and get medical attention. |  |  |  |  |
| 5. FIRE FIGHTING MEASURES |  |  |  |  |  |  |  |
| EXTINGUISHING MEDIA: |  |  | Use: Water spray, fog or mist. Alcohol resistant foam. Carbon dioxide (CO2). Dry |  |  |  |  |
|  |  |  | chemicals, sand, dolomite etc. |  |  |  |  |
| SPECIAL FIRE FIGHTING PROCEDURES: |  |  | Move container from fire area if it can be done without risk. Containers close to fire |  |  |  |  |
|  |  |  | should be removed or cooled with water. Cool containers exposed to flames with |  |  |  |  |
|  |  |  | water until well after the fire is out. Keep run-off water out of sewers and water |  |  |  |  |
|  |  |  | sources. Dike for water control. If risk of water pollution occurs, notify appropriate |  |  |  |  |
|  |  |  | authorities. |  |  |  |  |
| UNUSUAL FIRE & EXPLOSION HAZARDS: |  |  |  |  |  |  |  |
|  |  |  | HIGHLY FLAMMABLE! Vapours are heavier than air and may spread near ground to |  |  |  |  |
|  |  |  | sources of ignition. Vapour explosion and poison hazard indoors, outdoors and in |  |  |  |  |
|  |  |  | sewers. |  |  |  |  |
| HAZARDOUS DECOMPOSITION PRODUCTS: |  |  |  |  |  |  |  |
|  |  |  | Fire creates: Carbon monoxide (CO). Carbon dioxide (CO2) |  |  |  |  |
| 6. ACCIDENTAL RELEASE MEASURES |  |  |  |  |  |  |  |
| SPILL CLEANUP METHODS: |  |  | Stop leak if possible without risk. Extinguish all ignition sources. Avoid sparks, |  |  |  |  |
|  |  |  | flames, heat and smoking. Ventilate. Provide ventilation and confine spill. Do not |  |  |  |  |
|  |  |  | allow runoff to sewer. Absorb in vermiculite, dry sand or earth into containers. Avoid |  |  |  |  |
|  |  |  | contact with skin or inhalation of spillage, dust or vapour. Clean-up personnel |  |  |  |  |
|  |  |  | should use respiratory and/or liquid contact protection. |  |  |  |  |
| 7.HANDLING AND STORAGE |  |  |  |  |  |  |  |
| USAGE PRECAUTIONS: |  |  | Static electricity and formation of sparks must be prevented. Keep away from heat, |  |  |  |  |
|  |  |  | sparks and open flame. Use explosion proof electric equipment. Avoid spilling, skin |  |  |  |  |
|  |  |  | and eye contact. Ventilate well, avoid breathing vapours. Use approved respirator if |  |  |  |  |
|  |  |  | air contamination is above accepted level. Wear full protective clothing for prolonged |  |  |  |  |
|  |  |  | exposure and/or high concentrations. Do not us in confined spaces without |  |  |  |  |
|  |  |  | adequate ventilation and/or respirator. Use explosion proof electric equipment. |  |  |  |  |
| STORAGE PRECAUTIONS: |  |  | Flammable/combustible - Keep away from oxidisers, heat and flames. Keep in cool, |  |  |  |  |
|  |  |  | dry, ventilated storage and closed containers. May attack some plastics, rubber and |  |  |  |  |
|  |  |  | coatings. Aluminium is not suitable. Ground container and transfer equipment to |  |  |  |  |
|  |  |  | eliminate static electric sparks. |  |  |  |  |
| STORAGE CRITERIA: |  |  | Flammable liquid storage. |  |  |  |  |
| 8. EXPOSURE CONTROLS/PERSONAL PROTECTION |  |  |  |  |  |  |  |
| Ingredient name |  |  | CAS No. | STD | LT Exp | ST Exp |  |
|  |  |  |  |  | (8 hrs) | (15 mins) |  |
| Ethanol |  |  | 64-17-5 | OES | 1000ppm | No. std |  |
| Methanol |  |  | 67-56-1 | OES | 200ppm(Sk) | 250ppm(Sk) |  |
| INGREDIENT COMMENTS: |  |  | OES = Occupational Exposure Standard |  |  |  |  |
| PROTECTIVE EQUIPMENT: |  |  |  |  |  |  |  |
| VENTILATION: |  |  | Provide adequate general and local exhaust ventilation. Must not be handled in |  |  |  |  |
|  |  |  | confined space without sufficient ventilation. Provide sufficient venyilation during |  |  |  |  |
|  |  |  | operations which cause vapour formation. |  |  |  |  |
| RESPIRATORS: |  |  | Respiratory protection must be used if air concentraion exceeds acceptable level. If |  |  |  |  |
|  |  |  | ventilation is insufficient, suitable respiratory protection must be provided. Gas |  |  |  |  |
|  |  |  | cartridge (organic substances). |  |  |  |  |
| PROTECTIVE GLOVES: |  |  | Use suitable protective gloves if risk of skin contact. Use protective gloves made of: |  |  |  |  |
|  |  |  | Butyl rubber. Nitrile. |  |  |  |  |
| EYE PROTECTION: |  |  | Wear approved chemical safety goggles where eye exposure is reasonably |  |  |  |  |
|  |  |  | probable. |  |  |  |  |
| OTHER PROTECTION: |  |  | Wear appropriate clothing to prevent any possibility of liquid contact and repeated |  |  |  |  |
|  |  |  | or prolonged vapour contact. Provide eyewash station. |  |  |  |  |
| HYGIENIC WORK ROUTINES: |  |  | DO NOT SMOKE IN WORK AREA! Wash at the end of each work shift and before |  |  |  |  |
|  |  |  | eating, smoking and using the toilet. Promptly remove any clothing that becomes |  |  |  |  |
|  |  |  | contaminated. No eating or drinking while working with this material. |  |  |  |  |
| 9. PHYSICAL AND CHEMICAL PROPERTIES |  |  |  |  |  |  |  |
| APPEARANCE: |  |  | Liquid |  |  |  |  |
| COLOUR: |  |  | As ordered (Blue, white, red, green or yellow) |  |  |  |  |
| ODOUR/TASTE: |  |  | Alcohol |  |  |  |  |
| SOLUBILITY DESCRIPTION: |  |  | Soluble in water. Miscible with: Organic solvents (most). |  |  |  |  |
| BOILING POINT (°C): |  |  | 78 (for pure ethanol) @760mmHg |  |  |  |  |
| MELTING POINT (°C): |  |  | -11 (for pure ethanol) |  |  |  |  |
| SPECIFIC GRAVITY (WATER=1): |  |  | 0.789 @ 20°C |  |  |  |  |
| VAPOUR DENSITY (AIR=1): |  |  | 1.59 (pure ethanol) |  |  |  |  |
| VAPOUR PRESSURE: |  |  | 5.81 (pure ethanol)kPa @ 20°C |  |  |  |  |
| EVAPORATION RATE: |  |  | 3.4 (BuAc = 1) |  |  |  |  |
| VOLATILE BY VOL. (%): |  |  | 100 |  |  |  |  |
| VISCOSITY: |  |  | 1.19 cps@20°C |  |  |  |  |
| FLASHPOINT (°C): |  |  | 16 |  |  |  |  |
| FLASHPOINT METHOD: |  |  | CC (closed cup) |  |  |  |  |
| AUTO IGNITION TEMPERATURE (°C): |  |  | 365 |  |  |  |  |
| FLAMMABILITY LIMIT (LOWER %): |  |  | 3.5 |  |  |  |  |
| FLAMMABILITY LIMIT (UPPER %): |  |  | 19 |  |  |  |  |
| PARTITION COEFF. (WATER/OCTANOL): |  |  | 0.32 |  |  |  |  |
| 10. STABILITY AND REACTIVITY |  |  |  |  |  |  |  |
| STABILITY: |  |  | Avoid: Heat, sparks and flames. |  |  |  |  |
| MATERIALS TO AVOID: |  |  | Strong oxidising agents. Strong acids. |  |  |  |  |
| HAZARDOUS DECOMPOSITION PRODUCTS: |  |  |  |  |  |  |  |
|  |  |  | Fire creates: Vapours/gases/fumes of: Carbon monoxide (CO), Carbon |  |  |  |  |
|  |  |  | dioxide (CO2). |  |  |  |  |
| 11. TOXICOLOGICAL INFORMATION |  |  |  |  |  |  |  |
| TOXIC DOSE 1-LD50: |  |  | 7060mg/kg (oral rat) |  |  |  |  |
| HEALTH WARNINGS: |  |  | Gas or vapour is harmful on prolonged exposure or in high concentration. Irritant of |  |  |  |  |
|  |  |  | eyes and mucous membranes. Toxic through skin absorption (percutaneous). |  |  |  |  |
|  |  |  | Narcotic effect. Known or suspected teratogen. CNS depressant. Repeated |  |  |  |  |
|  |  |  | exposure may cause chronic eye irritation. High concentrations may cause severe |  |  |  |  |
|  |  |  | lung damage. Defatting, drying and cracking of skin. Swallowing concentrated |  |  |  |  |
|  |  |  | chemical may cause severe internal injury. Unconsciousness, death. |  |  |  |  |
| ROUTE OF ENTRY: |  |  | Inhalation. Ingestion. Skin and/or eye contact. |  |  |  |  |
| TARGET ORGANS: |  |  | Central nervous system. Eyes. Gastro-intestinal tract. Liver. Respiratory system, |  |  |  |  |
|  |  |  | lungs. Skin. |  |  |  |  |
| MEDICAL SYMPTOMS: |  |  | Irritation of eyes and mucous membranes. Dilated pupils. Rhinitis (inflammation of |  |  |  |  |
|  |  |  | the nasal mucous membranes). Upper respiratory irritation. General respiratory |  |  |  |  |
|  |  |  | distress, unproductive cough. May cause suffocation. Nausea, vomiting. Central |  |  |  |  |
|  |  |  | nervous system depression. Drowsiness, dizziness, disorientation, vertigo. |  |  |  |  |
|  |  |  | Headache. Behavioural changes. Hypotension (low blood pressure). Dizziness. |  |  |  |  |
|  |  |  | Confusion, agitation and/or excitation. |  |  |  |  |
| MEDICAL CONSIDERATIONS: |  |  | Convulsive disorders, CNS problems. |  |  |  |  |
| 12. ECOLOGICAL INFORMATION |  |  |  |  |  |  |  |
| LC50 96 HRS FISH (mg/Kg): |  |  | 14200-15400 fathead minnows (for ethanol) |  |  |  |  |
| EC50 48 DAPHNIA (mg/l): |  |  | 9000 (for ethanol) |  |  |  |  |
| IC50 72 ALGAE (mg/l): |  |  | 5000 (for ethanol) |  |  |  |  |
| ENVIRONMENTAL HAZARDS: |  |  | Bacterial inbition tests show that the material is not inhibitory to biomass. |  |  |  |  |
| MOBILITY: |  |  | This product will dissolve rapidly in water. If released to soil it will evaporate at a |  |  |  |  |
|  |  |  | rapid rate. The product is poorly absorbed onto soils or sediments. |  |  |  |  |
| BIO ACCUMULATION: |  |  | Product is not expected to bioaccumulate. |  |  |  |  |
| DEGRADABILITY: |  |  | The product is readily biodegradable. BOD5=37.74% of ThOD. BOD20=75.84& of |  |  |  |  |
|  |  |  | ThOD. Considered by the United Nations as least important in the formation of |  |  |  |  |
|  |  |  | episodic ozone. |  |  |  |  |
| 13. DISPOSAL |  |  |  |  |  |  |  |
| DISPOSAL METHODS: |  |  | Dispose of in accordance with Local Authority requirements. Confirm disposal |  |  |  |  |
|  |  |  | procedures with environmental enginner and local regulations. |  |  |  |  |
| 14. TRANSPORT INFORMATION |  |  |  |  |  |  |  |
| LABEL FOR CONVEYANCE: |  |  |  |  |  |  |  |
| ROAD: |  |  |  |  |  |  |  |
| UN No: |  |  | 1170 |  | ADR CLASS No: |  | 3 |
| HAZRD CLASS (ADR): |  |  | Class 3: Flammable liquids |  | ADR Label No: |  | 3 |
| HAZCHEM CODE: |  |  | 2YE |  |  |  |  |
| PROPER SHIPPING NAME: |  |  | Ethanol solution |  |  |  |  |
| RAIL: |  |  |  |  |  |  |  |
| RAIL TRANSPORT CLASS No: |  |  | 3 |  | RAILROAD PT: |  | II |
| SEA: |  |  |  |  |  |  |  |
| UN SEA: |  |  | 1170 |  | SEA TRANSPORT CLASS No: |  | 3 |
| SEA PACK GR: |  |  | II |  | EmS No: |  | 3-06 |
| MARINE POLLUTANT: |  |  | No |  |  |  |  |
| AIR: |  |  |  |  |  |  |  |
| UN AIR: |  |  | 1170 |  | AIR TRANSPORT CLASS No: |  | 3 |
| AIR PACK GR: |  |  | II |  |  |  |  |
| 15. REGULATORY INFORMATION |  |  |  |  |  |  |  |
| LABEL FOR SUPPLY: |  |  |  |  |  |  |  |
| RISK PHRASES: |  |  | R-11 | Highly Flammable. |  |  |  |
|  |  |  | R-20/21/22 | Harmful by inhalation, in contact with skin and if swallowed. |  |  |  |
|  |  |  | R-68/20/21/22 | Harmful: possible risk of irreversible effects through inhalation, in |  |  |  |
|  |  |  |  | contact with skin and if swallowed. |  |  |  |
| SAFETY PHRASES: |  |  | S-9 | Keep container in a well ventilated place. |  |  |  |
|  |  |  | S-16 | Keep away from sources of ignition - No Smoking. |  |  |  |
|  |  |  | S-51 | Use only in well ventilated areas. |  |  |  |
|  |  |  | S-36/37 | Wear suitable protective clothing and gloves. |  |  |  |
| UK REGULATORY REFERENCES: |  |  | Health & Safety at Work Act 1974 |  |  |  |  |
|  |  |  | The Control of Substances Hazardous to Health Regulations 1988 |  |  |  |  |
|  |  |  | Chemicals (Hazard Information & Packaging) Regulations 1993 |  |  |  |  |
| UK ENVIRONMENTAL LISTINGS: |  |  | Environmental Protection Act 1990 |  |  |  |  |
| GUIDANCE NOTES: |  |  | Occupational Exposure Limits EH40 |  |  |  |  |