1.PRODUCT / COMPANY IDENTIFICATION

Product Code.		05290

Revision Date.		13/07/2004

Company Name:		CONCEPT OILS,

UNIT 18 DROME ROAD

DEESIDE

FLINTSHIRE

CH5 2NY

Day time tel No.		01244 280999

Emergency Telephone No:	01925 636191

Product Name:		KUNAR 5

2.COMPOSITION / INFORMATION ON PRINCIPAL INGREDIENTS

Ingredient		% conc.	Classification			CAS		EINECS

Highly refined mineral oil    >90%	Xn, dead fish: R65, 51/53	64742-80-9	265-183-3

Triglyceride vegetable oil   >10%		not classified			8002-13-9

Highly refined mineral oil    >10%	not classified			64742-65-0	265-169-7

## 3 HAZARDS IDENTIFICATION

Classification:

This product is classified as dangerous according to the Dangerous Substances Directive 67/548/EEC

Classification / Symbol			St Andrews Cross, Dead Fish & Tree

Risk Phrases					R65

R51/53

Health Hazard

- Harmful, may cause lung damage if swallowed

- Repeated exposure may cause skin dryness and cracking

Environmental Hazards

- Non-toxic to aquatic organisms, may cause long term adverse effects in the aquatic environment

Physical / Chemical Hazards // Fire & Explosion Hazards

- Moderate hazard. Liquids can release vapours that can readily form flammable mixtures at temperatures at or above the flash point

## 4 FIRST AID MEASURES

## Exposure route			Symptoms				Treatment

## Inhalation		Irritation of throat, coughing			Remove to fresh air.

### Skin Contact		None				Wash with soap and water

## Eye Contact		Irritation, pain, redness		Wash with copious amounts

of water

## Ingestion		Nausea				DO NOT INDUCE VOMITING

Wash out mouth with water and seek

medical advice

## 5 FIRE FIGHTING MEASURES

# Suitable extinguishing media

Dry Powder, Water Fog, CO2 and Foam  DO NOT USE WATER.

special exposure hazards

The substances arising from the thermal decomposition of this product will depend largely upon the conditions bringing about decomposition. Any of the following may be expected.

Carbon Dioxide		Polycyclic Aromatic Hydrocarbons

Carbon Monoxide		Unburnt Hydrocarbons

Water				Unidentified Organic and Inorganic Compounds

Particulate Matter		Hydrogen Sulphide

special protective equipment

For large fires consider fire tunics (EN469), wet leg trousers (EN469), Wellington boots (EN345 Part ll 1996), helmet (EN443), flash-hood (EN531), gloves (EN323), self-contained breathing apparatus (EN137), self-contained breathing apparatus with airline attachment (EN139) .

## 6 ACCIDENTAL RELEASE MEASURES

Personal Precautions

Not classified as flammable but will support combustion. Remove sources of ignition.

Protective equipment may include PVC, Neoprene or nitrile rubber gloves; rubber knee-length safety boots and PVC jacket and trousers. Avoid contact with eyes and skin.

environmental precautions

If the product has contaminated any land it may require excavation of contaminated soil. If the product has entered a water-course or stream use absorbent booms to prevent further contamination.

##### Cleanup Procedure

Large spills should be bunded with sand or earth. The liquid should be reclaimed directly or in an absorbent medium and then transferred to clearly marked containers and disposed of in accordance with local by-laws and the requirements of the Environmental Protection Act.

Small spills should be absorbed in a suitable material and disposed of as for large spills.

## 7 HANDLING AND STORAGE

handling

Impervious gloves and overalls where regular contact is likely, and goggles if there is a risk of splashing.

Recommended Procedures – avoid prolonged contact with skin

storage

Keep at temperatures not exceeding 30 -40C. Protect from extremes of temperature, and protect from ingress of contaminants by keeping the contained closed or by maintaining a product seal around the cap

## 8 EXPOSURE CONTROLS

u.k. occupational exposure standards 2002

5 mg/M3 8 hour TWA value

10 mg/M3 15 min TWA value

Reference should be made to the HSE’s publication Methods for the Determination of Hazardous Substances (MDHS) 84 – Measurement of Oil Mists form mineral based metalworking fluids.

Respiratory protection (eg breathing apparatus or fume extraction) may be required when handling heated material. Half masks (EN149) or valved half masks (EN405) in combination with type A2 (EN141) and P2/3 (EN143) pre-filters may be considered when the liquid is at ambient temperatures, when at elevated temperatures then consider half masks (EN149) or valved half masks (EN405) in combination with type AX (EN371) and P2/3 (EN143) pre-filters.

Where repeated hand contact is likely wear suitable impervious gloves and wash with soap and water. Suitable gloves are Nitrile.

Any contaminated clothing should be removed and laundered before reuse.

Where there is a risk of splashing, suitable goggles should be worn conforming to BS EN 166 345B

#### Environmental controls

Users should be aware of environmental considerations and their duties under the Environmental Protection Act. Further advice may be found on various government websites eg www.dti.gov.uk/access/index.htm and www.envirowise.gov.uk

## 9 PHYSICAL AND CHEMICAL PROPERTIES

Appearance:				Pale amber liquid

Odour:					Perceptible

Acidity/Alkalinity				not applicable

Initial boiling point			>320C

Pour Point				-9C

Flash Point				>200C

Vapour pressure @20C		<0.1 k Pa

Relative density @ 15C			0.847

Solubility in water			insoluble

Viscosity @ 40C			4.8cSt

## 10 STABILITY AND REACTIVITY

Stability

Stable in normal use

conditions to avoid

Avoid high temperatures and strong oxidizing agents.

hazardous decomposition products

See section 5- Special exposure hazards.

## 11 TOXICOLOGICAL INFORMATION

ocular

Not expected to be irritant, may cause some discomfort.

dermal

Not expected to be irritant.

inhalation

Inhalation of mists or vapours under normal conditions is not likely to present any hazard , however at elevated temperatures inhalation of mists or vapours may cause respiratory irritation.

ingestion

Not expected to be toxic. DO NOT INDUCE VOMITING

## 12 ECOLOGICAL INFORMATION

air

The product is a mixture of non-volatile components which are not expected to be released to air in any significant quantities.

water

The product will form a floating layer on the surface and its components will not evaporate or dissolve to any great extent. Dissolved components will be absorbed in sediments. In aerobic waters and sediments they will biodegrade slowly, but in anaerobic conditions they will persist. The product is practically non-toxic to aquatic organisms but contains components which have a high potential to bio-accumulate.

# soil

Small volumes released on land will be absorbed in the upper soil layers and be biodegraded slowly. Larger volumes may penetrate into anaerobic soil layers in which the product will persist and may reach the water table on which it will form a floating layer. The more soluble components may dissolve but their high soil absorption coefficient and low solubility will prevent significant contamination of ground water.

## 13 DISPOSAL CONSIDERATIONS

Substance	via authorised waste disposal contractor to an approved waste disposal facility observing all local and national regulations

Container	As substance

## 14 TRANSPORT INFORMATION

Not dangerous for conveyance

UN Number

Primary Hazard

Subsidiary Hazard

Packing Group

Emergency Action Code

Proper shipping name

Class

## 15 REGULATORY INFORMATION

Supply label details	Ref CHIP 3

Label Name		Kunar 5 (contains Gas Oil - unspecified

#### Symbols		St Andrews Cross; Dead Fish and Tree

#### Risk Phrases		R65		Harmful; may cause lung damage if swallowed

R51/53		Toxic to aquatic organisms, may cause long term

adverse effects in the aquatic environment

#### Safety Phrases		S24		Avoid contact with skin

###### S62		If swallowed do not induce vomiting; seek medical

advice immediately and show this label or container

S61		Avoid release to the environment. Refer to Special

Instructions /  Safety Data Sheet

## 16 OTHER INFORMATION

Users should be trained in good industrial hygiene practices.